1. Platform Overview

Build a working prototype that:

- Reads data from motor sensors (current, voltage, temperature, vibration, RPM)

- Detects faults (overload, overheat, abnormal vibration, etc.)

- Controls a relay for safety shutdown

- Sends sensor data and alerts to the cloud (MQTT)

- Provides a web dashboard for real-time monitoring

2. Demo System Architecture

Block Diagram:

Code

[Sensors] → [ESP32 MCU] → [Relay/Contactor] → [Motor]

↓           ↑

[WiFi/MQTT]     |

↓           |

[Cloud Broker] ← [Web Dashboard]

- MCU: ESP32 Dev board

- Sensors: Current (ACS712), Temperature (DS18B20), Vibration (SW-420), RPM (optical encoder)

- Relay: 5V module

- Connectivity: WiFi (MQTT to broker)

- Cloud: Public MQTT broker (e.g., test.mosquitto.org), or ThingsBoard IoT

- Dashboard: Node.js/React web app

3. Example BOM (Bill of Materials)

| Item | Model/Type | Qty |
| --- | --- | --- |
| MCU Board | ESP32 Dev Board | 1 |
| Current Sensor | ACS712 5A | 1 |
| Temp Sensor | DS18B20 | 1 |
| Vibration Sensor | SW-420 | 1 |
| RPM Sensor | TCRT5000/Encoder | 1 |
| Relay Module | 5V Single Channel | 1 |
| Generic Motor | DC/AC Motor | 1 |
| Jumper Wires | - | -- |
| Breadboard/PCB | - | 1 |

4. Example Embedded Code (ESP32, Arduino IDE)

SmartMotorNode.ino

#include <WiFi.h>

#include <PubSubClient.h>

#include <OneWire.h>

#include <DallasTemperature.h>

// WiFi credentials

5. Cloud Demo: MQTT Dashboard

Try ThingsBoard Free Cloud (https://demo.thingsboard.io) or a public MQTT dashboard.

How it works:

- ESP32 publishes data (temperature, current, vibration, rpm) to topic motor/demo

- Publishes alerts to motor/demo/alert

- Dashboard subscribes to these topics and displays graphs, status, and alerts

Example Web Dashboard (React/Node.js) Outline

- Live graph for all sensors

- Status: RUNNING / STOPPED

- Fault log list (overheat, overload, vibration)

- Manual motor ON/OFF button (publish command topic)

- Historical data (from server DB)

6. External Dashboard Demo Example (ThingsBoard)

- Create a device called “MotorNodeDemo”

- Use “Integration → MQTT” for live data upload

- Use their ready-made widgets for sensor display and alerts

7. Extension Ideas

- Add SMS/email notifications using cloud server or Twilio API.

- Use a real industrial CAN/Modbus bus for communication.

- Add predictive maintenance AI (using Azure IoT/ML).

- Connect multiple nodes for monitoring a whole plant.

8. Demo Workflow

- Startup: ESP32 connects to WiFi, broker.

- Sampling: Every second, reads sensors.

- Publishing: All sensor data to MQTT/cloud.

- Safety: Motor stop relay triggered by any threshold breach, alert published.

- Dashboard: Updates status, graphs data, shows alerts.

9. Repo Structure Example

Code

smart-motor-monitoring-demo/

├── firmware/

│   └── SmartMotorNode.ino

├── dashboard/

│   ├── src/

│   │   └── (React components)

│   └── server/

│       └── app.js (Node.js MQTT subscriber)

└── docs/

├── architecture.md

└── schematics.png

10. Next Steps

- You can try the firmware code above on ESP32

- Set up ThingsBoard demo cloud or run a local MQTT broker

- Connect the dashboard front-end to MQTT using mqtt.js in Node.js or use Python for a proof-of-concept