// INFRA-NEX PHASE-2 - RELAY MOTOR CONTROL PERFECTED

// Relay HIGH = Motor ON (Active High Logic)

#include <WiFi.h>

#include "ThingSpeak.h"

#include <Wire.h>

#include <Adafruit_INA219.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#include <math.h>

/* ================= CONFIG ================= */

#define THINGSPEAK_CHANNEL_ID   3008248

#define THINGSPEAK_API_KEY      "MC8F2GPRVGC3KZIC"

const char* WIFI_SSID = "vivo Y200 5G";

const char* WIFI_PASS = "priyanknh";

/* =============== PINS =================== */

#define PIN_DS18B20   4

#define PIN_ADXL_X    34

#define PIN_ADXL_Y    35

#define PIN_ADXL_Z    32

#define PIN_RPM       27

#define PIN_ZMPT      33

#define PIN_RELAY     25   // RELAY HIGH = MOTOR ON

#define LED_GREEN   2

#define LED_YELLOW  14

#define LED_RED     26

/* ============= THRESHOLDS =============== */

const float TEMP_WARN = 70.0;   const float TEMP_WARN_HYST = 65.0;

const float TEMP_CRIT = 90.0;   const float TEMP_CRIT_HYST = 85.0;

const float VIB_WARN = 0.35;    const float VIB_WARN_HYST = 0.30;

const float VIB_CRIT = 0.50;    const float VIB_CRIT_HYST = 0.45;

const float CURR_WARN = 4.5;    const float CURR_WARN_HYST = 4.0;

const float CURR_CRIT = 6.0;    const float CURR_CRIT_HYST = 5.5;

const float RPM_LOW = 1300;     const float RPM_LOW_HYST = 1250;

const float RPM_HIGH = 1700;    const float RPM_HIGH_HYST = 1750;

/* ============= RELAY MOTOR CONTROL ========== */

bool motorOn = false;                     // Motor state

unsigned long lastRelayToggle = 0;        // Debounce timer

const unsigned long RELAY_COOLDOWN = 5000;      // 5s between toggles

const unsigned long MOTOR_START_DELAY = 30000;  // 30s startup grace

/* ============= SENSORS ==================== */

OneWire oneWire(PIN_DS18B20);

DallasTemperature tempSensor(&oneWire);

Adafruit_INA219 ina219;                   // FIX: default ctor, no Wire argument [web:15][web:25]

WiFiClient client;

/* ============= RPM COUNTER ================ */

volatile uint32_t rpmPulses = 0;

unsigned long lastRpmCalc = 0;

float currentRpm = 0;

void IRAM_ATTR rpmPulse() {

rpmPulses++;

}

/* ============= DATA BUFFERS =============== */

#define WINDOW_SIZE 10

float tempHistory[WINDOW_SIZE], vibHistory[WINDOW_SIZE];

float currHistory[WINDOW_SIZE], rpmHistory[WINDOW_SIZE];

uint8_t bufferPos = 0;

/* ============= FAULT STATUS =============== */

uint8_t systemFault = 0;  // 0=OK, 1=Warning, 2=Critical, 3=Predictive

/* ================ CORE FUNCTIONS ================ */

// FIX: argument is ADC reading, not pin number

float accelerationG(int adcValue) {

float voltage = (adcValue * 3.3f) / 4095.0f;

return (voltage - 1.65f) / 0.33f;  // ADXL345 sensitivity (approx)

}

float movingAverage(float* buffer) {

float total = 0;

for (int i = 0; i < WINDOW_SIZE; i++) {

total += buffer[i];

}

return total / WINDOW_SIZE;

}

bool isTrendRising(float* buffer) {

return buffer[bufferPos] > buffer[(bufferPos + WINDOW_SIZE - 1) % WINDOW_SIZE];

}

void connectNetwork() {

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\n✓ WiFi Connected - IP: " + WiFi.localIP().toString());

}

/* ========== MOTOR RELAY PERFECTION ========== */

/**

* INDUSTRIAL-GRADE RELAY CONTROL

* ✅ Active HIGH logic (HIGH = MOTOR ON)

* ✅ 5s cooldown prevents relay wear

* ✅ Hysteresis eliminates chatter

* ✅ 30s startup grace period

* ✅ Safe emergency shutdown

*/

void controlMotorRelay(bool turnOnRequested) {

unsigned long now = millis();

// SAFETY: Startup grace - force ON for first 30 seconds

if (now < MOTOR_START_DELAY) {

turnOnRequested = true;

Serial.println("🚀 MOTOR STARTUP GRACE PERIOD");

}

// DEBOUNCE: Prevent rapid toggling

if (now - lastRelayToggle < RELAY_COOLDOWN) {

return;  // Ignore during cooldown

}

// RELAY LOGIC WITH HYSTERESIS

bool finalDecision = turnOnRequested;

if (turnOnRequested && !motorOn) {

// Starting motor: Relaxed thresholds OK

finalDecision = (systemFault <= 1);  // Warnings OK to start

} else if (!turnOnRequested && motorOn) {

// Stopping motor: Strict thresholds only

finalDecision = (systemFault >= 2);  // Critical required to stop

}

if (finalDecision != motorOn) {

motorOn = finalDecision;

digitalWrite(PIN_RELAY, motorOn ? HIGH : LOW);

lastRelayToggle = now;

Serial.printf("🔌 MOTOR %s | Fault Level: %d | Time: %lu\n",

motorOn ? "ON" : "OFF", systemFault, now / 1000);

}

}

void updateStatusLEDs() {

digitalWrite(LED_GREEN, LOW);

digitalWrite(LED_YELLOW, LOW);

digitalWrite(LED_RED, LOW);

if (systemFault == 0) digitalWrite(LED_GREEN, HIGH);

else if (systemFault == 1) digitalWrite(LED_YELLOW, HIGH);

else digitalWrite(LED_RED, HIGH);

}

bool isHealthy(bool useHysteresis) {

float tAvg = movingAverage(tempHistory);

float vAvg = movingAverage(vibHistory);

float cAvg = movingAverage(currHistory);

float rAvg = movingAverage(rpmHistory);

float tWarn = useHysteresis ? TEMP_WARN_HYST : TEMP_WARN;

float tCrit = useHysteresis ? TEMP_CRIT_HYST : TEMP_CRIT;

float vWarn = useHysteresis ? VIB_WARN_HYST : VIB_WARN;

float vCrit = useHysteresis ? VIB_CRIT_HYST : VIB_CRIT;

float cWarn = useHysteresis ? CURR_WARN_HYST : CURR_WARN;

float cCrit = useHysteresis ? CURR_CRIT_HYST : CURR_CRIT;

float rLow = useHysteresis ? RPM_LOW_HYST : RPM_LOW;

float rHigh = useHysteresis ? RPM_HIGH_HYST : RPM_HIGH;

// CRITICAL CHECKS

if (tAvg > tCrit || vAvg > vCrit || cAvg > cCrit) return false;

// WARNING CHECKS

if (tAvg > tWarn || vAvg > vWarn ||

rAvg < rLow || rAvg > rHigh) return false;

return true;

}

/* ================= SETUP ================== */

void setup() {

Serial.begin(115200);

Serial.println("\n=== INFRA-NEX PHASE-2 MOTOR CONTROL ===");

// RELAY SAFETY INIT

pinMode(PIN_RELAY, OUTPUT);

digitalWrite(PIN_RELAY, LOW);  // MOTOR OFF

// SENSOR PINS

pinMode(PIN_RPM, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(PIN_RPM), rpmPulse, FALLING);

// LED INDICATORS

pinMode(LED_GREEN, OUTPUT);

pinMode(LED_YELLOW, OUTPUT);

pinMode(LED_RED, OUTPUT);

// I2C SENSORS

Wire.begin();

if (!ina219.begin()) {                  // uses default I2C addr and Wire [web:15][web:25]

Serial.println("❌ INA219 Current Sensor Missing!");

}

tempSensor.begin();

// NETWORK

connectNetwork();

ThingSpeak.begin(client);               // official pattern [web:10]

Serial.println("✅ SYSTEM READY - RELAY HIGH = MOTOR ON");

}

/* ================= MAIN LOOP ================ */

void loop() {

static unsigned long lastScan = 0;

unsigned long now = millis();

if (now - lastScan < 5000) return;      // 5s cycle

lastScan = now;

/* === READ SENSORS === */

float busCurrent = ina219.getCurrent_mA() / 1000.0f;  // A (from mA) [web:15][web:25]

tempSensor.requestTemperatures();

float motorTemp = tempSensor.getTempCByIndex(0);

int ax = analogRead(PIN_ADXL_X);

int ay = analogRead(PIN_ADXL_Y);

int az = analogRead(PIN_ADXL_Z);

float vibration = sqrtf(

sq(accelerationG(ax)) +

sq(accelerationG(ay)) +

sq(accelerationG(az))

);

/* === CALCULATE RPM === */

if (now - lastRpmCalc > 1000) {

noInterrupts();

uint32_t pulses = rpmPulses;

rpmPulses = 0;

interrupts();

currentRpm = pulses * 60.0f;  // RPM calculation (pulses per second * 60)

lastRpmCalc = now;

}

/* === UPDATE BUFFERS === */

tempHistory[bufferPos] = motorTemp;

vibHistory[bufferPos] = vibration;

currHistory[bufferPos] = busCurrent;

rpmHistory[bufferPos] = currentRpm;

bufferPos = (bufferPos + 1) % WINDOW_SIZE;

/* === PREDICTIVE ANALYTICS === */

bool tempRising = isTrendRising(tempHistory);

bool vibRising  = isTrendRising(vibHistory);

bool currRising = isTrendRising(currHistory);

/* === FAULT CLASSIFICATION === */

bool healthyNormal = isHealthy(false);

bool healthyHyst   = isHealthy(true);

if (tempRising && vibRising && currRising &&

movingAverage(tempHistory) > TEMP_WARN &&

movingAverage(currHistory) > CURR_WARN) {

systemFault = 3;  // PREDICTIVE

} else if (!healthyNormal) {

systemFault = 2;  // CRITICAL

} else if (!healthyHyst) {

systemFault = 1;  // WARNING

} else {

systemFault = 0;  // PERFECT

}

/* === RELAY MOTOR CONTROL === */

bool motorShouldRun = (systemFault < 2);  // Stop only CRITICAL

controlMotorRelay(motorShouldRun);

updateStatusLEDs();

/* === CLOUD TELEMETRY === */

ThingSpeak.setField(1, (float)busCurrent);          // explicit type [web:20][web:10]

ThingSpeak.setField(2, (float)motorTemp);

ThingSpeak.setField(3, (float)vibration);

ThingSpeak.setField(4, (long)currentRpm);           // RPM fits in long

ThingSpeak.setField(5, (int)systemFault);

ThingSpeak.setField(6, (int)(motorOn ? 1 : 0));

ThingSpeak.setField(7, (long)(now / 1000));         // uptime seconds, avoid ambiguity [web:20]

ThingSpeak.setField(8, (long)WiFi.RSSI());

int cloudStatus = ThingSpeak.writeFields(THINGSPEAK_CHANNEL_ID, THINGSPEAK_API_KEY);

/* === MONITORING DISPLAY === */

Serial.println("═" + String(50, '═'));

Serial.printf("⚡ Current: %.2fA | 🔥 Temp: %.1f°C | 📳 Vib: %.3fg | ⚙️ RPM: %.0f\n",

busCurrent, motorTemp, vibration, currentRpm);

Serial.printf("📊 Averages: T=%.1f°C I=%.2fA V=%.3fg R=%.0f\n",

movingAverage(tempHistory), movingAverage(currHistory),

movingAverage(vibHistory), movingAverage(rpmHistory));

Serial.printf("🚦 FAULT: %d | 🏭 MOTOR: %s | 📡 Cloud: %s\n",

systemFault, motorOn ? "ON" : "OFF",

cloudStatus == 200 ? "OK" : "ERROR");

}