#include <Wire.h>

#include <Adafruit_INA219.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#include <WiFi.h>

#include <HTTPClient.h>

// =====================================================

// WIFI + THINGSPEAK CONFIG

// =====================================================

const char* WIFI_SSID     = "Prazal's Motorola ";

const char* WIFI_PASSWORD = "22112211";

const char* THINGSPEAK_API_KEY = "7DF6OSIMH0V3YCKR";   // ThingSpeak Write API Key

const char* THINGSPEAK_HOST    = "http://api.thingspeak.com/update";

const unsigned long WIFI_RECONNECT_INTERVAL   = 10000;  // try reconnect every 10s if dropped

const unsigned long THINGSPEAK_UPLOAD_INTERVAL = 16000; // ThingSpeak min is 15s, 16s for safety

unsigned long wifiRetryTimer = 0;

unsigned long thingSpeakTimer = 0;

// =====================================================

// GPIO MAP

// =====================================================

const int ADXL_X_PIN   = 34;

const int ADXL_Y_PIN   = 35;

const int ADXL_Z_PIN   = 32;

const int ZMPT_PIN     = 33;

const int HALL_PIN     = 27;

const int ONE_WIRE_BUS = 04;

const int I2C_SDA_PIN  = 21;

const int I2C_SCL_PIN  = 22;

const int RELAY_PIN    = 25;

// =====================================================

// RELAY LOGIC

// =====================================================

const bool RELAY_ACTIVE_LOW = true;

// =====================================================

// SENSOR CALIBRATION CONSTANTS (unchanged from your sensor code)

// =====================================================

const float MAGNETS_PER_REV = 2.0;

const int ADXL_X_ZERO = 1860;

const int ADXL_Y_ZERO = 1865;

const int ADXL_Z_ZERO = 1890;

const float ADXL_SENSITIVITY = 373.0;

const int   ZMPT_SAMPLES     = 400;

const float ZMPT_CALIBRATION = 0.5;

const int   ESP32_DC_OFFSET  = 2048;

const unsigned long READ_INTERVAL = 500;

// =====================================================================

// FAULT THRESHOLDS

// =====================================================================

const float RPM_MIN_RUN        = 300.0;

const float RPM_MAX_RUN        = 900.0;

const float CURR_TRIP_mA       = 60.0;

const float CURR_LOW_mA        = 15.0;

const float VIB_TRIP_g         = 0.900;

const float DC_VOLT_MIN        = 10.0;

const float DC_VOLT_MAX        = 15.0;

const float AC_RMS_MIN         = 220.0;

const float AC_RMS_MAX         = 340.0;

const float TEMP_MAX_C         = 40.0;

// =====================================================================

// =====================================================

// TIMING (debounce + auto-restart)

// =====================================================

const unsigned long FAULT_CONFIRM_MS = 3000;

const unsigned long STARTUP_GRACE_MS = 1000;

const unsigned long RESTART_DELAY_MS = 2000;

// =====================================================

// GLOBALS — sensors

// =====================================================

Adafruit_INA219 ina219;

bool ina219Found = false;

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

DeviceAddress sensorAddress;

bool tempSensorFound = false;

unsigned long readTimer = 0;

volatile unsigned long lastPulseTime = 0;

volatile unsigned long pulseInterval = 0;

volatile bool newPulseDetected = false;

bool relayState = false;

bool conversionInProgress = false;

unsigned long conversionStartTime = 0;

unsigned long conversionDelayMs = 188;

float lastValidTempC = DEVICE_DISCONNECTED_C;

// -------- Latest readings cache (used by ThingSpeak uploader) --------

float g_currentRPM   = 0;

float g_vibrationG   = 0;

float g_current_mA   = 0;

float g_acVoltageRMS = 0;

float g_motorVolts   = 0;

float g_tempC        = 0;

bool  g_relayIsOn    = false;

int   g_faultCode    = 0;

uint8_t g_healthScore = 100;  // NEW: Health score cache

// =====================================================

// GLOBALS — fault logic / state machine

// =====================================================

enum SysState { RUNNING, TRIPPED_WAIT };

SysState state = RUNNING;

unsigned long stateChangeTime = 0;

unsigned long motorStartTime  = 0;

struct FaultTimer { bool active = false; unsigned long start = 0; };

FaultTimer rpmLowFault, rpmHighFault, currHighFault, currLowFault, vibFault;

FaultTimer dcLowFault, dcHighFault, acLowFault, acHighFault, tempFault;

enum FaultCode { FC0_NORMAL, FC1_JAM, FC2_BEARING, FC3_OVERHEAT, FC4_SLIP,

FC5_SUPPLY, FC6_LOOSENESS, FC7_RUNAWAY };

const char* faultName(FaultCode fc) {

switch (fc) {

case FC1_JAM:       return "FC1-MechanicalJam";

case FC2_BEARING:   return "FC2-BearingFailure";

case FC3_OVERHEAT:  return "FC3-WindingOverheat";

case FC4_SLIP:      return "FC4-BeltSlip";

case FC5_SUPPLY:    return "FC5-SupplyFault";

case FC6_LOOSENESS: return "FC6-Looseness";

case FC7_RUNAWAY:   return "FC7-Runaway";

default:            return "FC0-Normal";

}

}

void updateTimer(FaultTimer &f, bool bad, unsigned long now) {

if (bad) { if (!f.active) { f.active = true; f.start = now; } }

else { f.active = false; }

}

bool isConfirmed(FaultTimer &f, unsigned long now) {

return f.active && (now - f.start >= FAULT_CONFIRM_MS);

}

// =====================================================

// HEALTH SCORE CALCULATION (from health_score_calc.v)

// =====================================================

uint8_t calculateHealthScore(bool rpm_stall, bool rpm_over, bool rpm_degraded,

bool volt_bad,

bool high_curr, bool low_curr,

bool high_temp, bool high_vib) {

int16_t deduction = 0;

// RPM health tier

if (rpm_stall || rpm_over)      deduction += 20;

else if (rpm_degraded)          deduction += 10;

// Voltage health

if (volt_bad)                   deduction += 20;

// Current health tier

if (high_curr)                  deduction += 20;

else if (low_curr)              deduction += 10;

// Temperature health

if (high_temp)                  deduction += 20;

// Vibration health

if (high_vib)                   deduction += 20;

// Score saturation: deduction >= 100 forces health to 0

if (deduction >= 100) return 0;

return (uint8_t)(100 - deduction);

}

// =====================================================

// ISR

// =====================================================

void IRAM_ATTR countPulse() {

unsigned long currentMicros = micros();

if (currentMicros - lastPulseTime > 20000) {

pulseInterval = currentMicros - lastPulseTime;

lastPulseTime = currentMicros;

newPulseDetected = true;

}

}

// =====================================================

// RELAY HELPERS

// =====================================================

void setRelay(bool turnOn) {

relayState = turnOn;

if (RELAY_ACTIVE_LOW) digitalWrite(RELAY_PIN, turnOn ? LOW : HIGH);

else digitalWrite(RELAY_PIN, turnOn ? HIGH : LOW);

}

void relayOn()  { setRelay(true); }

void relayOff() { setRelay(false); }

// =====================================================

// SENSOR HELPERS

// =====================================================

float readZMPT_RMS() {

long long sumSquares = 0;

for (int i = 0; i < ZMPT_SAMPLES; i++) {

int raw = analogRead(ZMPT_PIN) - ESP32_DC_OFFSET;

sumSquares += (long)raw * raw;

delayMicroseconds(40);

}

float rms = sqrt((float)sumSquares / ZMPT_SAMPLES);

return rms * ZMPT_CALIBRATION;

}

float readRPM() {

static float currentRPM = 0.0;

if (micros() - lastPulseTime > 2000000) {

currentRPM = 0.0;

} else if (newPulseDetected) {

noInterrupts();

unsigned long intervalCopy = pulseInterval;

newPulseDetected = false;

interrupts();

if (intervalCopy > 0) {

currentRPM = (60000000.0 / intervalCopy) / MAGNETS_PER_REV;

}

}

return currentRPM;

}

void startTemperatureConversion() {

if (tempSensorFound && !conversionInProgress) {

sensors.requestTemperatures();

conversionInProgress = true;

conversionStartTime = millis();

}

}

void updateTemperatureIfReady() {

if (!tempSensorFound || !conversionInProgress) return;

if (millis() - conversionStartTime >= conversionDelayMs) {

float tempC = sensors.getTempCByIndex(0);

lastValidTempC = (tempC != DEVICE_DISCONNECTED_C) ? tempC : DEVICE_DISCONNECTED_C;

conversionInProgress = false;

}

}

// =====================================================

// WIFI HELPERS (non-blocking)

// =====================================================

void connectWiFi() {

Serial.print("Connecting to WiFi: ");

Serial.println(WIFI_SSID);

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

unsigned long startAttempt = millis();

while (WiFi.status() != WL_CONNECTED && millis() - startAttempt < 10000) {

delay(250);   // only blocking spot, limited to setup() and max 10s

Serial.print(".");

}

if (WiFi.status() == WL_CONNECTED) {

Serial.print("\nWiFi connected. IP: ");

Serial.println(WiFi.localIP());

} else {

Serial.println("\nWiFi connect failed, will retry in background.");

}

}

void maintainWiFi(unsigned long now) {

if (WiFi.status() != WL_CONNECTED && now - wifiRetryTimer >= WIFI_RECONNECT_INTERVAL) {

wifiRetryTimer = now;

Serial.println("WiFi disconnected, retrying...");

WiFi.disconnect();

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

}

}

// =====================================================

// THINGSPEAK UPLOAD (non-blocking, rate-limited)

// =====================================================

void uploadToThingSpeak() {

if (WiFi.status() != WL_CONNECTED) return;

if (String(THINGSPEAK_API_KEY) == "PASTE_YOUR_WRITE_API_KEY_HERE") return; // no key set yet

HTTPClient http;

String url = String(THINGSPEAK_HOST) + "?api_key=" + THINGSPEAK_API_KEY +

"&field1=" + String(g_currentRPM, 1) +

"&field2=" + String(g_vibrationG, 3) +

"&field3=" + String(g_current_mA, 1) +

"&field4=" + String(g_acVoltageRMS, 1) +

"&field5=" + String(g_motorVolts, 2) +

"&field6=" + String(g_tempC, 1) +

"&field7=" + String(g_relayIsOn ? 1 : 0) +

"&field8=" + String(g_faultCode) +

"&field9=" + String(g_healthScore);  // NEW: Health score in field9

http.begin(url);

int httpCode = http.GET();

if (httpCode > 0) {

Serial.print("ThingSpeak upload OK, entry #");

Serial.println(http.getString());

} else {

Serial.print("ThingSpeak upload failed, error: ");

Serial.println(http.errorToString(httpCode));

}

http.end();

}

// =====================================================

// SETUP

// =====================================================

void setup() {

Serial.begin(115200);

pinMode(HALL_PIN, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(HALL_PIN), countPulse, FALLING);

pinMode(RELAY_PIN, OUTPUT);

relayOff();

Wire.begin(I2C_SDA_PIN, I2C_SCL_PIN);

ina219Found = ina219.begin();

if (ina219Found) {

ina219.setCalibration_32V_2A();

Serial.println("INA219: OK");

} else {

Serial.println("INA219: NOT FOUND");

}

sensors.begin();

int deviceCount = sensors.getDeviceCount();

Serial.print("DS18B20 devices found: "); Serial.println(deviceCount);

if (deviceCount > 0 && sensors.getAddress(sensorAddress, 0)) {

sensors.setResolution(sensorAddress, 10);

sensors.setWaitForConversion(false);

conversionDelayMs = 750 / (1 << (12 - sensors.getResolution(sensorAddress)));

tempSensorFound = true;

startTemperatureConversion();

Serial.println("DS18B20: OK");

} else {

Serial.println("DS18B20: NOT FOUND / ADDRESS FAIL");

}

analogReadResolution(12);

connectWiFi();

Serial.println("=== INFRA-NEX THRESHOLD + FAULT LOGIC + HEALTH SCORE + CLOUD STARTED ===");

readTimer = millis();

state = RUNNING;

motorStartTime = millis();

}

// =====================================================

// LOOP

// =====================================================

void loop() {

unsigned long now = millis();

updateTemperatureIfReady();

maintainWiFi(now);

if (now - readTimer >= READ_INTERVAL) {

readTimer = now;

// -------- Relay drive based on state --------

bool relayIsOn;

if (state == RUNNING) {

relayOn();

relayIsOn = true;

} else {

relayOff();

relayIsOn = false;

}

// -------- Read all sensors --------

float currentRPM = readRPM();

int rawX = analogRead(ADXL_X_PIN);

int rawY = analogRead(ADXL_Y_PIN);

int rawZ = analogRead(ADXL_Z_PIN);

float gX = (float)(rawX - ADXL_X_ZERO) / ADXL_SENSITIVITY;

float gY = (float)(rawY - ADXL_Y_ZERO) / ADXL_SENSITIVITY;

float gZ = (float)(rawZ - ADXL_Z_ZERO) / ADXL_SENSITIVITY;

float vibrationG = sqrt((gX * gX) + (gY * gY) + (gZ * gZ));

float current_mA = 0, busVoltage = 0, shuntVolts = 0, motorVolts = 0;

if (ina219Found) {

current_mA = ina219.getCurrent_mA();

busVoltage = ina219.getBusVoltage_V();

shuntVolts = ina219.getShuntVoltage_mV();

motorVolts = busVoltage + (shuntVolts / 1000.0);

}

float acVoltageRMS = readZMPT_RMS();

if (!conversionInProgress) startTemperatureConversion();

bool sensorFaultTemp = (lastValidTempC == DEVICE_DISCONNECTED_C);

// -------- Fault condition checks --------

bool pastStartupGrace = relayIsOn && (now - motorStartTime >= STARTUP_GRACE_MS);

bool rpmLowBad   = pastStartupGrace && (currentRPM < RPM_MIN_RUN);

bool rpmHighBad  = pastStartupGrace && (currentRPM > RPM_MAX_RUN);

bool currHighBad = pastStartupGrace && (current_mA > CURR_TRIP_mA);

bool currLowBad  = pastStartupGrace && (current_mA < CURR_LOW_mA);

bool vibBad      = relayIsOn && (vibrationG > VIB_TRIP_g);

bool dcLowBad    = relayIsOn && (motorVolts < DC_VOLT_MIN);

bool dcHighBad   = relayIsOn && (motorVolts > DC_VOLT_MAX);

bool acLowBad    = (acVoltageRMS < AC_RMS_MIN) && (acVoltageRMS > 0);

bool acHighBad   = (acVoltageRMS > AC_RMS_MAX);

bool tempBad     = relayIsOn && !sensorFaultTemp && (lastValidTempC > TEMP_MAX_C);

// -------- Debounce each channel --------

updateTimer(rpmLowFault,   rpmLowBad,   now);

updateTimer(rpmHighFault,  rpmHighBad,  now);

updateTimer(currHighFault, currHighBad, now);

updateTimer(currLowFault,  currLowBad,  now);

updateTimer(vibFault,      vibBad,      now);

updateTimer(dcLowFault,    dcLowBad,    now);

updateTimer(dcHighFault,   dcHighBad,   now);

updateTimer(acLowFault,    acLowBad,    now);

updateTimer(acHighFault,   acHighBad,   now);

updateTimer(tempFault,     tempBad,     now);

bool rpmLowC   = isConfirmed(rpmLowFault, now);

bool rpmHighC  = isConfirmed(rpmHighFault, now);

bool currHighC = isConfirmed(currHighFault, now);

bool currLowC  = isConfirmed(currLowFault, now);

bool vibC      = isConfirmed(vibFault, now);

bool dcLowC    = isConfirmed(dcLowFault, now);

bool dcHighC   = isConfirmed(dcHighFault, now);

bool acLowC    = isConfirmed(acLowFault, now);

bool acHighC   = isConfirmed(acHighFault, now);

bool tempOkC   = isConfirmed(tempFault, now);

// -------- Multi-sensor correlation -> Fault Code --------

FaultCode fc = FC0_NORMAL;

if      (currHighC && rpmLowC)                    fc = FC1_JAM;

else if (vibC && rpmLowC)                         fc = FC2_BEARING;

else if (tempOkC && currHighC)                    fc = FC3_OVERHEAT;

else if (rpmLowC && !currHighC)                   fc = FC4_SLIP;

else if (dcLowC || dcHighC || acLowC || acHighC)  fc = FC5_SUPPLY;

else if (vibC && !rpmLowC)                        fc = FC6_LOOSENESS;

else if (currLowC && rpmHighC)                    fc = FC7_RUNAWAY;

bool anyFaultConfirmed = (fc != FC0_NORMAL);

// -------- State machine: trip -> wait 5s -> auto-restart --------

if (state == RUNNING && anyFaultConfirmed) {

state = TRIPPED_WAIT;

stateChangeTime = now;

relayOff();

relayIsOn = false;

Serial.print(">>> FAULT TRIPPED: "); Serial.println(faultName(fc));

} else if (state == TRIPPED_WAIT && (now - stateChangeTime >= RESTART_DELAY_MS)) {

Serial.println(">>> Restart delay elapsed -> retrying motor <<<");

state = RUNNING;

motorStartTime = now;

}

// -------- NEW: HEALTH SCORE CALCULATION --------

// Derive boolean flags matching dashboard thresholds

bool rpm_stall    = rpmLowC;           // RPM < 300

bool rpm_over     = rpmHighC;          // RPM > 900

bool rpm_degraded = false;             // Could add intermediate RPM check here if needed

bool volt_bad     = dcLowC || dcHighC;  // DC < 10V or > 15V

bool high_curr    = currHighC;          // Current > 60mA

bool low_curr     = currLowC;           // Current < 15mA

bool high_temp    = tempOkC;            // Temp > 40°C

bool high_vib     = vibC;               // Vibration > 0.9g

uint8_t healthScore = calculateHealthScore(

rpm_stall, rpm_over, rpm_degraded,

volt_bad, high_curr, low_curr, high_temp, high_vib

);

// -------- Cache latest values for cloud upload --------

g_currentRPM   = currentRPM;

g_vibrationG   = vibrationG;

g_current_mA   = current_mA;

g_acVoltageRMS = acVoltageRMS;

g_motorVolts   = motorVolts;

g_tempC        = sensorFaultTemp ? 0 : lastValidTempC;

g_relayIsOn    = relayIsOn;

g_faultCode    = (int)fc;

g_healthScore  = healthScore;  // NEW: Cache health score

// -------- Status print --------

Serial.print("State: "); Serial.print(state == RUNNING ? "RUNNING" : "TRIPPED_WAIT");

Serial.print(" | Fault: "); Serial.print(faultName(fc));

Serial.print(" | Health: "); Serial.print(healthScore); Serial.print("/100");  // NEW

Serial.print(" | Relay: "); Serial.print(relayIsOn ? "ON" : "OFF");

Serial.print(" | RPM: "); Serial.print(currentRPM, 1);

Serial.print(" | Temp: ");

if (sensorFaultTemp) Serial.print("ERR");

else { Serial.print(lastValidTempC, 1); Serial.print("C"); }

Serial.print(" | DC: "); Serial.print(motorVolts, 2); Serial.print("V");

Serial.print(" | Current: "); Serial.print(current_mA, 1); Serial.print("mA");

Serial.print(" | AC RMS: "); Serial.print(acVoltageRMS, 1); Serial.print("V");

Serial.print(" | Vib: "); Serial.print(vibrationG, 3); Serial.println("g");

Serial.print(" | WiFi: "); Serial.println(WiFi.status() == WL_CONNECTED ? "OK" : "DOWN");

}

// -------- ThingSpeak upload (separate, rate-limited timer) --------

if (now - thingSpeakTimer >= THINGSPEAK_UPLOAD_INTERVAL) {

thingSpeakTimer = now;

uploadToThingSpeak();

}

}