// INFRA-NEX: ESP32 Dev Module demo code

// Features: reads 6 sensors, basic fault logic, MQTT publish, relay control, LED fault detection

#include <WiFi.h>

#include <PubSubClient.h>

#include <Wire.h>

#include <Adafruit_INA219.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#include <math.h>  // Added for sqrt()

// ---------- USER CONFIG ----------

// Wi-Fi

const char* WIFI_SSID     = "Jayshree krishna";

const char* WIFI_PASSWORD = "Shreenathg@131";

// MQTT

const char* MQTT_BROKER   = "broker.emqx.io";

const uint16_t MQTT_PORT  = 1883;

const char* MQTT_CLIENTID = "infra_nex_esp32_1";

const char* MQTT_TOPIC    = "infra_nex/motor1/data";

const char* MQTT_ALERT    = "infra_nex/motor1/alert";

// Pin mapping (change as per your wiring)

#define PIN_DS18B20_BUS   4     // OneWire bus for both DS18B20

#define PIN_ADXL_X        34    // ADXL335 X (analog)

#define PIN_ADXL_Y        35    // ADXL335 Y (analog)

#define PIN_ADXL_Z        32    // ADXL335 Z (analog)

#define PIN_TCRT_RPM      27    // TCRT5000 digital output

#define PIN_ZMPT_VOLTAGE  33    // ZMPT101B analog output (via conditioning)

#define PIN_RELAY         25    // Relay control pin

#define PIN_LED_NORMAL    2     // Onboard LED - Green/Normal (GPIO2 safe for most ESP32 boards) [web:16]

#define PIN_LED_WARNING   14    // External Yellow LED - Warning (GPIO14 safe) [web:9]

#define PIN_LED_CRITICAL  26    // External Red LED - Critical (GPIO26 safe) [web:17]

// RPM measurement

volatile uint32_t pulseCount = 0;

uint32_t lastRPMMillis = 0;

float currentRPM = 0;

// DS18B20

OneWire oneWire(PIN_DS18B20_BUS);

DallasTemperature sensors(&oneWire);

// INA219 (I2C default pins on ESP32: SDA=21, SCL=22)

Adafruit_INA219 ina219;

// WiFi & MQTT clients

WiFiClient espClient;

PubSubClient mqttClient(espClient);

// ---------- THRESHOLDS (EXAMPLE ONLY – ADJUST BY TESTING) ----------

float TH_TEMP_WARNING   = 70.0;   // °C

float TH_TEMP_CRITICAL  = 90.0;   // °C

float TH_VIB_WARNING    = 0.35;   // g (approx from ADXL values)

float TH_VIB_CRITICAL   = 0.50;   // g

float TH_VOLT_LOW       = 190.0;  // V (for 230 V system, example)

float TH_VOLT_HIGH      = 250.0;  // V

float TH_RPM_LOW        = 1300;   // example for 1500 rpm motor

float TH_RPM_HIGH       = 1700;

float TH_CURRENT_HIGH   = 6.0;    // A (example)

// LED blink timing

unsigned long ledBlinkTime = 0;

bool ledState = false;

uint8_t blinkPattern = 0;  // 0=NORMAL, 1=WARNING(slow blink), 2=CRITICAL(fast blink)

// Utility: connect Wi-Fi

void connectWiFi() {

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

}

}

// Utility: connect MQTT

void connectMQTT() {

mqttClient.setServer(MQTT_BROKER, MQTT_PORT);

while (!mqttClient.connected()) {

mqttClient.connect(MQTT_CLIENTID);

delay(500);

}

}

// Interrupt for RPM pulses (TCRT5000)

void IRAM_ATTR rpmISR() {

pulseCount++;

}

// Convert ADXL335 reading to approx g (simple, needs calibration)

float adxlToG(int raw) {

// For 3.3 V, 12-bit ADC (0–4095), ADXL335 ~330 mV/g, 0g ~ Vcc/2

// This is a rough approximation; do real calibration in lab.

float voltage = (raw / 4095.0) * 3.3;

float zeroG   = 3.3 / 2.0;

float g = (voltage - zeroG) / 0.33;

return g;

}

// Update LED fault indication

void updateFaultLEDs(bool warning, bool critical) {

unsigned long now = millis();

if (critical) {

// Fast blink red LED (200ms ON/OFF) [web:15]

if (now - ledBlinkTime >= 200) {

ledState = !ledState;

digitalWrite(PIN_LED_CRITICAL, ledState ? HIGH : LOW);

ledBlinkTime = now;

blinkPattern = 2;

}

// Turn off others

digitalWrite(PIN_LED_NORMAL, LOW);

digitalWrite(PIN_LED_WARNING, LOW);

}

else if (warning) {

// Slow blink yellow LED (500ms ON/OFF) [web:15]

if (now - ledBlinkTime >= 500) {

ledState = !ledState;

digitalWrite(PIN_LED_WARNING, ledState ? HIGH : LOW);

ledBlinkTime = now;

blinkPattern = 1;

}

// Turn off others

digitalWrite(PIN_LED_NORMAL, LOW);

digitalWrite(PIN_LED_CRITICAL, LOW);

}

else {

// Steady green onboard LED for NORMAL [web:15]

digitalWrite(PIN_LED_NORMAL, HIGH);

digitalWrite(PIN_LED_WARNING, LOW);

digitalWrite(PIN_LED_CRITICAL, LOW);

blinkPattern = 0;

}

}

void setup() {

Serial.begin(115200);

// Initialize relay

pinMode(PIN_RELAY, OUTPUT);

digitalWrite(PIN_RELAY, LOW);  // motor ON (assuming LOW = ON, change if opposite)

// Initialize RPM interrupt

pinMode(PIN_TCRT_RPM, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(PIN_TCRT_RPM), rpmISR, FALLING);

// Initialize LED pins (safe GPIOs selected) [web:9][web:13]

pinMode(PIN_LED_NORMAL, OUTPUT);

pinMode(PIN_LED_WARNING, OUTPUT);

pinMode(PIN_LED_CRITICAL, OUTPUT);

// Initial state: all LEDs OFF

digitalWrite(PIN_LED_NORMAL, LOW);

digitalWrite(PIN_LED_WARNING, LOW);

digitalWrite(PIN_LED_CRITICAL, LOW);

// Initialize sensors

Wire.begin();

if (!ina219.begin()) {

Serial.println("Failed to find INA219 chip");

}

sensors.begin();

connectWiFi();

connectMQTT();

lastRPMMillis = millis();

Serial.println("INFRA-NEX Motor Monitoring Started with LED Fault Detection");

}

void loop() {

if (!mqttClient.connected()) {

connectMQTT();

}

mqttClient.loop();

// Update LED status every 50ms for smooth blinking

static uint32_t lastLEDUpdate = 0;

if (millis() - lastLEDUpdate >= 50) {

// LED logic will be updated in main sample loop below

lastLEDUpdate = millis();

}

// 1 sample per second (approx)

static uint32_t lastSampleMillis = 0;

if (millis() - lastSampleMillis >= 1000) {

lastSampleMillis = millis();

// ---- Read current & bus voltage from INA219 ----

float shunt_mV   = ina219.getShuntVoltage_mV();

float bus_V      = ina219.getBusVoltage_V();

float load_V     = bus_V + (shunt_mV / 1000.0);

float current_mA = ina219.getCurrent_mA();

float current_A  = current_mA / 1000.0;

// ---- Read DS18B20 temperatures ----

sensors.requestTemperatures();

float temp1 = sensors.getTempCByIndex(0);  // motor body

float temp2 = sensors.getTempCByIndex(1);  // bearing

if (temp1 == DEVICE_DISCONNECTED_C || temp2 == DEVICE_DISCONNECTED_C) {

temp1 = -999; temp2 = -999;  // Sensor fault handling

}

// ---- Read ADXL335 for vibration ----

int rawX = analogRead(PIN_ADXL_X);

int rawY = analogRead(PIN_ADXL_Y);

int rawZ = analogRead(PIN_ADXL_Z);

float gX = adxlToG(rawX);

float gY = adxlToG(rawY);

float gZ = adxlToG(rawZ);

// simple overall vibration magnitude

float vibMag = sqrt(gX * gX + gY * gY + gZ * gZ);

// ---- Read ZMPT101B for RMS voltage (approx) ----

// Simplified: just read one sample and scale; for real RMS use many samples + calibration

int rawVolt = analogRead(PIN_ZMPT_VOLTAGE);

float vInst = (rawVolt / 4095.0) * 3.3;  // sensor output

// Apply your calibration factor:

float lineVoltage = vInst * 100.0;       // placeholder scale, FIX IN LAB

// ---- Compute RPM from pulse count ----

uint32_t now = millis();

uint32_t dt = now - lastRPMMillis;

if (dt > 0) {

noInterrupts();

uint32_t pulses = pulseCount;

pulseCount = 0;

interrupts();

// If 1 pulse per revolution:

float seconds = dt / 1000.0;

currentRPM = (pulses / seconds) * 60.0;

lastRPMMillis = now;

}

// ---- Fault detection logic ----

bool warning = false;

bool critical = false;

String faultText = "NORMAL";

// Temperature faults

if (temp1 > TH_TEMP_CRITICAL || temp2 > TH_TEMP_CRITICAL) {

critical = true;

faultText = "OVER_TEMPERATURE";

} else if (temp1 > TH_TEMP_WARNING || temp2 > TH_TEMP_WARNING) {

warning = true;

faultText = "HIGH_TEMPERATURE";

}

// Vibration faults

if (vibMag > TH_VIB_CRITICAL) {

critical = true;

faultText = "HIGH_VIBRATION";

} else if (vibMag > TH_VIB_WARNING && !critical) {

warning = true;

faultText = "VIBRATION_WARNING";

}

// Voltage faults

if (lineVoltage < TH_VOLT_LOW || lineVoltage > TH_VOLT_HIGH) {

warning = true;

if (!critical) faultText = "VOLTAGE_ABNORMAL";

}

// RPM faults

if (currentRPM < TH_RPM_LOW || currentRPM > TH_RPM_HIGH) {

warning = true;

if (!critical) faultText = "RPM_DEVIATION";

}

// Current faults

if (current_A > TH_CURRENT_HIGH) {

critical = true;

faultText = "OVER_CURRENT";

}

// Update LED fault indication [web:15]

updateFaultLEDs(warning, critical);

// Decide motor control

if (critical) {

digitalWrite(PIN_RELAY, HIGH); // assume HIGH = OFF

} else {

digitalWrite(PIN_RELAY, LOW);  // motor ON

}

// ---- Build JSON payload ----

String payload = "{";

payload += "\"current_A\":" + String(current_A, 3) + ",";

payload += "\"voltage_V\":" + String(lineVoltage, 1) + ",";

payload += "\"temp_body_C\":" + String(temp1, 1) + ",";

payload += "\"temp_bearing_C\":" + String(temp2, 1) + ",";

payload += "\"vib_g\":" + String(vibMag, 3) + ",";

payload += "\"rpm\":" + String(currentRPM, 1) + ",";

payload += "\"fault\":\"" + faultText + "\",";

payload += "\"led_status\":\"" + String(blinkPattern) + "\"";  // Added LED status

payload += "}";

// ---- Publish to MQTT ----

mqttClient.publish(MQTT_TOPIC, payload.c_str());

// Send separate alert message if warning/critical

if (warning || critical) {

mqttClient.publish(MQTT_ALERT, faultText.c_str());

}

// Debug output

Serial.println("Status: " + faultText + " | RPM: " + String(currentRPM,0) +

" | Temp: " + String(temp1,1) + "/" + String(temp2,1) +

" | Current: " + String(current_A,2) + "A");

}

}