Smart Motor Monitoring & Safety Node — Architecture

1. Physical Layer

Sensors

- Current Sensor (e.g., ACS712, Hall effect sensor)
Measures motor current, detects overload, short-circuit, etc.

- Voltage Sensor (e.g., ZMPT101B)
Monitors supply voltage for undervoltage/overvoltage conditions.

- Temperature Sensor (e.g., PT100, DS18B20)
Tracks motor body/winding temperature for thermal protection.

- Vibration Sensor (e.g., MEMS accelerometer, piezo sensor)
Detects abnormal vibrations—early sign of bearing issues or imbalance.

- Speed/Rotation Sensor (e.g., optical encoder, Hall effect, tachometer)
Records motor speed and RPM.

- Environmental Sensors (optional)
Monitors ambient humidity, dust, etc., for harsh situations.

Actuators

- Relay/Contactor
For motor ON/OFF control & emergency shutdown.

2. Embedded Node Hardware

- Microcontroller/Edge Device
E.g., ESP32, STM32, Arduino, Raspberry Pi Pico, etc.

  - Reads sensor data

  - Controls relays/actuators

  - Runs onboard analytics (fault detection, trending)

  - Supports connectivity (WiFi/Ethernet/Bluetooth/LoRa/CAN)

  - Security features (secure boot, encrypted communication)

- Power Supply and Protection

  - Isolation circuits

  - TVS diodes, fuses for protection

  - Battery backup (optional)

3. Firmware/Software Layer

- Data Acquisition Module

  - Scheduled and event-driven sensor polling

  - Signal conditioning (filtering, amplification)

- Data Processing Module

  - Threshold-based fault detection (overcurrent, overtemp, etc.)

  - Machine learning algorithms for predictive maintenance (optional)

  - Vibration analysis for early warning

- Control Logic

  - Emergency shutdown routines

  - Manual/remote start/stop

- Communication Stack

  - MQTT, HTTP REST API, Modbus TCP/Serial, or custom protocol

  - Local dashboard display (optional LCD/OLED/Web UI)

  - Push notifications for critical alerts

4. Connectivity/Networking Layer

- Local/Remote Connectivity

  - WiFi/Ethernet for cloud/local server connection

  - Bluetooth/Zigbee for on-site diagnostics

  - LoRa/CAN for industrial long-range networking

- Cloud Gateway

  - Secure connection to cloud platform (AWS IoT, Azure IoT, custom server)

  - For data logging, remote control, historical analysis

5. Cloud/Server/Edge Platform

- Data Storage & Analytics

  - Real-time dashboard for motor health/status

  - Historical trends, charts, and predictive alerts

  - Fault/event logs

- User Interface

  - Web/mobile app for monitoring & management

  - Alert configuration, report downloads

- Integration APIs

  - REST endpoints or OPC-UA for integration with SCADA, ERP, MES systems

- Security

  - End-to-end encryption

  - Authentication/authorization

6. Safety & Reliability Layer

- Failsafe Mechanisms

  - Watchdog timers

  - Redundant critical sensors

  - Local override/physical emergency stop

- Operational Safety

  - Isolation circuits for high-voltage

  - Ground fault detection

  - Auto-recovery features

Block Diagram

Here’s an outline in text:

[Sensors] → MCU/Edge Node → Actuators (Relay/Stop)
     |
     ↓
   Local Dashboard
     |
     ↓
  Wireless/Wired Communication → Cloud Platform
     |
     ↓
   Remote Monitoring Dashboard/App

Sample Stack

- Hardware: ESP32/STM32, industrial grade sensors, relay module

- Firmware: Embedded C/C++, MicroPython, FreeRTOS

- Network: MQTT (TLS), WiFi

- Cloud: AWS IoT / Azure IoT / Custom Node.js server

- Frontend: React Dashboard, Android/iOS App

- Integration: REST API/OPC-UA/SCADA system