// INFRA-NEX: ESP32 Motor Monitoring with ThingSpeak Cloud and Web Dashboard

// Features: 6 sensors, fault detection, ThingSpeak API upload, relay+LED control, Web Server with Dashboard/Graphs/Table

#include <WiFi.h>

#include "ThingSpeak.h"

#include <Wire.h>

#include <Adafruit_INA219.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#include <math.h>

#include <ESPAsyncWebServer.h>  // New: For web server

#include <AsyncTCP.h>           // Dependency

// ======================================

// THINGSPEAK CONFIG - UPDATE THESE!

// ======================================

#define THINGSPEAK_CHANNEL_ID     1234567UL

#define THINGSPEAK_WRITE_APIKEY   "MC8F2GPRVGC3KZIC"

// WiFi credentials

const char* WIFI_SSID     = "vivo Y200 5G";

const char* WIFI_PASSWORD = "priyanknh";

// Sensor pins (unchanged)

#define PIN_DS18B20     4

#define PIN_ADXL_X      34

#define PIN_ADXL_Y      35

#define PIN_ADXL_Z      32

#define PIN_TCRT_RPM    27

#define PIN_ZMPT_VOLT   33

#define PIN_RELAY       25

#define PIN_LED_GREEN   2

#define PIN_LED_YELLOW  14

#define PIN_LED_RED     26

// Thresholds (unchanged)

const float TEMP_WARN = 70.0, TEMP_CRIT = 90.0;

const float VIB_WARN = 0.35, VIB_CRIT = 0.50;

const float VOLT_LOW = 190.0, VOLT_HIGH = 250.0;

const float RPM_LOW = 1300, RPM_HIGH = 1700;

const float CURR_HIGH = 6.0;

// Global variables (unchanged)

volatile uint32_t rpmPulses = 0;

float currentRPM = 0;

uint32_t lastRPMTime = 0;

unsigned long ledTimer = 0;

bool ledState = false;

uint8_t faultLevel = 0;

OneWire oneWire(PIN_DS18B20);

DallasTemperature ds18b20(&oneWire);

Adafruit_INA219 ina219;

WiFiClient client;

// ---------- NEW: Web Server and Data Storage ----------

AsyncWebServer server(80);

// Struct for historical data (circular buffer)

#define MAX_HISTORY 50

struct SensorData {

float current_A, voltage_V, temp_body_C, temp_bearing_C, vib_g, rpm;

unsigned long timestamp;

};

SensorData history[MAX_HISTORY];

int historyIndex = 0;

int historyCount = 0;

// Performance variables

float avgCurrent = 0, avgVoltage = 0, avgTempBody = 0, avgTempBearing = 0, avgVib = 0, avgRPM = 0;

float minTempBody = 999, maxTempBody = -999, minRPM = 999, maxRPM = -999;

unsigned long uptimeMillis = 0;

String currentFault = "NORMAL";

bool relayState = false;

// HTML for dashboard (embedded)

const char index_html[] PROGMEM = R"rawliteral(

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="UTF-8">

<meta name="viewport" content="width=device-width, initial-scale=1.0">

<title>INFRA-NEX Motor Dashboard</title>

<link href="https://cdn.jsdelivr.net/npm/bootstrap@5.3.0/dist/css/bootstrap.min.css" rel="stylesheet">

<script src="https://cdn.jsdelivr.net/npm/chart.js"></script>

</head>

<body class="bg-light">

<div class="container mt-4">

<h1 class="text-center">INFRA-NEX Motor Monitoring Dashboard</h1>

<p class="text-center">ThingSpeak Channel: <a href="https://thingspeak.com/channels/1234567" target="_blank">View on ThingSpeak</a></p>

<div class="row">

<div class="col-md-4">

<div class="card">

<div class="card-header">Controls</div>

<div class="card-body">

<button id="toggleRelay" class="btn btn-primary">Toggle Relay</button>

<p id="relayStatus">Relay: OFF</p>

<p id="ledStatus">LED: Normal</p>

</div>

</div>

</div>

<div class="col-md-8">

<div class="card">

<div class="card-header">Performance Overview</div>

<div class="card-body">

<table class="table table-striped">

<thead><tr><th>Metric</th><th>Value</th></tr></thead>

<tbody>

<tr><td>Uptime</td><td id="uptime">0s</td></tr>

<tr><td>Fault Status</td><td id="fault">NORMAL</td></tr>

<tr><td>Avg Current (A)</td><td id="avgCurrent">0</td></tr>

<tr><td>Avg Voltage (V)</td><td id="avgVoltage">0</td></tr>

<tr><td>Avg Temp Body (°C)</td><td id="avgTempBody">0</td></tr>

<tr><td>Avg Temp Bearing (°C)</td><td id="avgTempBearing">0</td></tr>

<tr><td>Avg Vibration (g)</td><td id="avgVib">0</td></tr>

<tr><td>Avg RPM</td><td id="avgRPM">0</td></tr>

<tr><td>Min/Max Temp Body (°C)</td><td id="minMaxTemp">0/0</td></tr>

<tr><td>Min/Max RPM</td><td id="minMaxRPM">0/0</td></tr>

</tbody>

</table>

</div>

</div>

</div>

</div>

<div class="row mt-4">

<div class="col-md-6"><canvas id="currentChart"></canvas></div>

<div class="col-md-6"><canvas id="voltageChart"></canvas></div>

<div class="col-md-6"><canvas id="tempChart"></canvas></div>

<div class="col-md-6"><canvas id="vibChart"></canvas></div>

<div class="col-md-6"><canvas id="rpmChart"></canvas></div>

</div>

</div>

<script>

const charts = {

current: new Chart(document.getElementById('currentChart'), { type: 'line', data: { labels: [], datasets: [{ label: 'Current (A)', data: [], borderColor: 'blue' }] }, options: { responsive: true } }),

voltage: new Chart(document.getElementById('voltageChart'), { type: 'line', data: { labels: [], datasets: [{ label: 'Voltage (V)', data: [], borderColor: 'green' }] }, options: { responsive: true } }),

temp: new Chart(document.getElementById('tempChart'), { type: 'line', data: { labels: [], datasets: [{ label: 'Temp Body (°C)', data: [], borderColor: 'red' }, { label: 'Temp Bearing (°C)', data: [], borderColor: 'orange' }] }, options: { responsive: true } }),

vib: new Chart(document.getElementById('vibChart'), { type: 'line', data: { labels: [], datasets: [{ label: 'Vibration (g)', data: [], borderColor: 'purple' }] }, options: { responsive: true } }),

rpm: new Chart(document.getElementById('rpmChart'), { type: 'line', data: { labels: [], datasets: [{ label: 'RPM', data: [], borderColor: 'black' }] }, options: { responsive: true } })

};

setInterval(() => {

fetch('/data').then(res => res.json()).then(data => {

document.getElementById('uptime').textContent = data.uptime;

document.getElementById('fault').textContent = data.fault;

document.getElementById('avgCurrent').textContent = data.avgCurrent.toFixed(2);

document.getElementById('avgVoltage').textContent = data.avgVoltage.toFixed(2);

document.getElementById('avgTempBody').textContent = data.avgTempBody.toFixed(2);

document.getElementById('avgTempBearing').textContent = data.avgTempBearing.toFixed(2);

document.getElementById('avgVib').textContent = data.avgVib.toFixed(2);

document.getElementById('avgRPM').textContent = data.avgRPM.toFixed(2);

document.getElementById('minMaxTemp').textContent = `${data.minTempBody.toFixed(1)}/${data.maxTempBody.toFixed(1)}`;

document.getElementById('minMaxRPM').textContent = `${data.minRPM.toFixed(1)}/${data.maxRPM.toFixed(1)}`;

document.getElementById('relayStatus').textContent = `Relay: ${data.relayState ? 'ON' : 'OFF'}`;

document.getElementById('ledStatus').textContent = `LED: ${['Normal', 'Warning', 'Critical'][data.ledStatus]}`;

const time = new Date().toLocaleTimeString();

charts.current.data.labels.push(time); charts.current.data.datasets[0].data.push(data.current_A); charts.current.update();

charts.voltage.data.labels.push(time); charts.voltage.data.datasets[0].data.push(data.voltage_V); charts.voltage.update();

charts.temp.data.labels.push(time); charts.temp.data.datasets[0].data.push(data.temp_body_C); charts.temp.data.datasets[1].data.push(data.temp_bearing_C); charts.temp.update();

charts.vib.data.labels.push(time); charts.vib.data.datasets[0].data.push(data.vib_g); charts.vib.update();

charts.rpm.data.labels.push(time); charts.rpm.data.datasets[0].data.push(data.rpm); charts.rpm.update();

Object.values(charts).forEach(chart => {

if (chart.data.labels.length > 50) {

chart.data.labels.shift();

chart.data.datasets.forEach(ds => ds.data.shift());

}

});

});

}, 5000);

document.getElementById('toggleRelay').addEventListener('click', () => {

fetch('/toggle', { method: 'POST' }).then(() => alert('Relay toggled!'));

});

</script>

</body>

</html>

)rawliteral";

// Utility functions (unchanged)

void IRAM_ATTR rpmISR() { rpmPulses++; }

float adxlToG(int adcRaw) { /* unchanged */ }

void connectWiFi() { /* unchanged */ }

void updateLEDs() { /* unchanged */ }

// ---------- NEW: Web Server Handlers ----------

void setupWebServer() {

server.on("/", HTTP_GET, [](AsyncWebServerRequest *request) {

request->send_P(200, "text/html", index_html);

});

server.on("/data", HTTP_GET, [](AsyncWebServerRequest *request) {

String json = "{";

json += "\"current_A\":" + String(history[(historyIndex - 1 + MAX_HISTORY) % MAX_HISTORY].current_A, 3) + ",";

json += "\"voltage_V\":" + String(history[(historyIndex - 1 + MAX_HISTORY) % MAX_HISTORY].voltage_V, 1) + ",";

json += "\"temp_body_C\":" + String(history[(historyIndex - 1 + MAX_HISTORY) % MAX_HISTORY].temp_body_C, 1) + ",";

json += "\"temp_bearing_C\":" + String(history[(historyIndex - 1 + MAX_HISTORY) % MAX_HISTORY].temp_bearing_C, 1) + ",";

json += "\"vib_g\":" + String(history[(historyIndex - 1 + MAX_HISTORY) % MAX_HISTORY].vib_g, 3) + ",";

json += "\"rpm\":" + String(history[(historyIndex - 1 + MAX_HISTORY) % MAX_HISTORY].rpm, 1) + ",";

json += "\"fault\":\"" + currentFault + "\",";

json += "\"uptime\":\"" + String(uptimeMillis / 1000) + "s\",";

json += "\"avgCurrent\":" + String(avgCurrent, 2) + ",";

json += "\"avgVoltage\":" + String(avgVoltage, 2) + ",";

json += "\"avgTempBody\":" + String(avgTempBody, 2) + ",";

json += "\"avgTempBearing\":" + String(avgTempBearing, 2) + ",";

json += "\"avgVib\":" + String(avgVib, 2) + ",";

json += "\"avgRPM\":" + String(avgRPM, 2) + ",";

json += "\"minTempBody\":" + String(minTempBody, 1) + ",";

json += "\"maxTempBody\":" + String(maxTempBody, 1) + ",";

json += "\"minRPM\":" + String(minRPM, 1) + ",";

json += "\"maxRPM\":" + String(maxRPM, 1) + ",";

json += "\"relayState\":" + String(relayState) + ",";

json += "\"ledStatus\":" + String(faultLevel);

json += "}";

request->send(200, "application/json", json);

});

server.on("/toggle", HTTP_POST, [](AsyncWebServerRequest *request) {

relayState = !relayState;

digitalWrite(PIN_RELAY, relayState ? LOW : HIGH);

request->send(200, "text/plain", "Toggled");

});

server.begin();

}

void setup() {

Serial.begin(115200);

delay(1000);

pinMode(PIN_RELAY, OUTPUT);

digitalWrite(PIN_RELAY, LOW);

pinMode(PIN_TCRT_RPM, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(PIN_TCRT_RPM), rpmISR, FALLING);

pinMode(PIN_LED_GREEN, OUTPUT);

pinMode(PIN_LED_YELLOW, OUTPUT);

pinMode(PIN_LED_RED, OUTPUT);

Wire.begin();

if (!ina219.begin()) Serial.println("INA219 ERROR!");

ds18b20.begin();

connectWiFi();

ThingSpeak.begin(client);

setupWebServer();  // NEW

Serial.println("=== INFRA-NEX READY ===");

Serial.println("ThingSpeak Channel ID: " + String(THINGSPEAK_CHANNEL_ID));

Serial.println("Dashboard: http://" + WiFi.localIP().toString());

Serial.println("ThingSpeak: https://thingspeak.com/channels/" + String(THINGSPEAK_CHANNEL_ID));

}

void loop() {

static unsigned long lastUpdate = 0;

if (millis() - lastUpdate < 20000) {

updateLEDs();

return;

}

lastUpdate = millis();

uptimeMillis = millis();

// Sensor readings (unchanged)

float current_A = ina219.getCurrent_mA() / 1000.0;

float busV = ina219.getBusVoltage_V();

float voltage_V = busV + (ina219.getShuntVoltage_mV() / 1000);

ds18b20.requestTemperatures();

float tempBody = ds18b20.getTempCByIndex(0);

float tempBear = ds18b20.getTempCByIndex(1);

if (tempBody == DEVICE_DISCONNECTED_C) tempBody = -999;

if (tempBear == DEVICE_DISCONNECTED_C) tempBear = -999;

float gx = adxlToG(analogRead(PIN_ADXL_X));

float gy = adxlToG(analogRead(PIN_ADXL_Y));

float gz = adxlToG(analogRead(PIN_ADXL_Z));

float vibMag = sqrt(gx*gx + gy*gy + gz*gz);

float zmptRaw = analogRead(PIN_ZMPT_VOLT);

float lineVoltage = (zmptRaw / 4095.0 * 3.3) * 85.0;

unsigned long now = millis();

if (now - lastRPMTime > 1000) {

noInterrupts();

uint32_t pulses = rpmPulses;

rpmPulses = 0;

interrupts();

currentRPM = (pulses * 60.0) / ((now - lastRPMTime) / 1000.0);

lastRPMTime = now;

}

// Fault detection (unchanged)

faultLevel = 0;

String faultMsg = "NORMAL";

if (current_A > CURR_HIGH || tempBody > TEMP_CRIT || tempBear > TEMP_CRIT || vibMag > V